Electric Energy Audit

Take Home Project

Bring your completed Electric Energy Audit to class on Wednesday, 04 September

Learning Objectives: You will learn about your personal use of electric energy.

Background Information: The accompanying sheet lists the power consumed by typical appliances.

Instructions:

- Using the table on the reverse side, fill in the power consumption in Watts for active (or charging: column b), idle (energy save mode for the device: column e), and off (maintaining a vampire load: column h) for 10 appliances where you live.

- Write in the number of hours per week that the appliance is active (column c), idle (column f), or off (column i). If the device is unplugged, DO NOT include that in the “off” hours. The number of hours in a week is 168.

- Calculate the energy in kWh consumed by every appliance for each of its states of operation during a week. (For example: multiply column b by column c to get column d, Energy (kWh)).

- Sum each of the energy columns, specifically:

- Add the total energy consumed (in kWh) by all the appliances in their active state (column d).

- Add the total energy consumed (in kWh) by all the appliances in their idle state (column g).

- Add the total energy consumed (in kWh) by all the appliances in their off state (column j).

- Add the previous three energy totals to calculate the total energy consumed by the appliance in all states (column k).

- Energy consumed by appliances in their “off” state is wasted and could be eliminated by unplugging the appliance or by putting it on a power strip with an off-switch.

- Calculate the cost of the energy used (column l) by multiplying column k by $0.13/kWh (i.e., the cost of electricity in Richmond).

- Calculate the fraction (expressed as a percent) of the energy consumed by all of your appliances (together) that is wasted (use the equation below the table).

| (a) | (b) | (c) | (d) | (e) | (f) | (g) | (h) | (i) | (j) | (k)=(d)+(g)+(j) | (l)=(k)x$___ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Appliance | Active (W) | On-time (hrs) | Energy (kWh) | Idle (W) | Idle Time (hrs) | Energy (kWh) | “off” (W) | Off Time (hrs) | Energy (kWh) | Total Energy (kWh) | Cost ($) |
| Ex: Ipad | 6 | 21 | 0.126 | 3 | 35 | 0.105 | 0.4 | 112 | 0.045 | 0.276 |  |
| Total |  |  |  |  |  |  |  |  |  |  |  |

Fraction of energy lost due to vampire loads =    _          _Total “off” energy (kWh)_________ x 100% = _____________ %

Total Active (kWh) + Idle (kWh) + off (kWh)